Alcohol Gas Sensor

S1016 Alcohol Gas Sensor

Measuring range: 0 ~ 6000 ppm

Accuracy: ±3% F.S

Resolution: 1.5 ppm

The sensing component of Alcohol Gas Sensor is made of the metallic oxide semiconductor on the aluminum oxide plate. When there is alcohol gas exists in the environment, conductivity of sensor will be increasing with concentration of alcohol gas. The sensor converts changing of conductivity into concentration of alcohol gas.

Component

| No. | Model | Name | Picture |
| --- | --- | --- | --- |
| 1 | S1016 | Alcohol Gas Sensor |  |
| 2 |  | Alcohol Gas Sensor electrode |  |

Usage

- Connect Alcohol Gas Sensor to any port in SenseDisc.

- Place the Alcohol Gas Sensor into the place with alcohol gas to measure.

Typical experiment

Detect the content of alcohol gas

Notes

- Do not immerse the sensor into liquid alcohol. It will damage the probe.

- Preheating is required. The initial value could be wrong.